# Week 4: Chapter 7 & 8 Lab – Setting Up and Troubleshooting a Local Network

This lab requires the use of your Netlab Windows 10 VM

Make sure to reserve a FULL 2.5 hours for this lab and/or extend time aa necessary.

Step 1: Log into your Online NDG Netlab Account, create a reservation, and launch a VM:

- Navigate to: https://netlabve2.flc.losrios.edu/home.cgi

Username: EXACTLY the same as your Canvas username

- Username is case-sensitive – use lowercase letters ONLY

- Enter first initial + last name + last 3 digits of your student id

Password: 98repair (no spaces/case-sensitive)

Warning: Do NOT use the CAPS LOCK key when using the VMs.

If you are having trouble logging in, creating a reservation, entering your reservation, or logging into the VM student account, please review last week’s tutorial video:

Tutorial video: Using NDG NETLAB

AFTER ENTERING INTO YOUR WIN10 VM, ALWAYS CHANGE THESE PERFORMANCE SETTINGS BELOW **BEFORE** YOU START:

WARNING: This lab requires entering command-line commands.

Making TYPOS/MISTAKES when entering commands will cause the commands to FAIL – be VERY CAREFUL throughout this lab to enter commands EXACTLY how you see them.

VERIFY you see the expected result BEFORE continuing on to the next step and ALWAYS RERUN any commands you have entered incorrectly.

1. Custom Project 1: Viewing Network Connection Settings

Review pgs. 421-422 (item #3) for information on the ipconfig command. Review pgs. 337-339 for key terms such as IPv4/IPv6, subnet mask, default gateway, DNS/DHCP server.

Step 1: Enter cmd into the Windows Search bar located on the taskbar of your Windows 10 VM  click on Command Prompt App to open it.

Step 2: While the Command Prompt window is open, right-click on its icon on the taskbar and click pin to taskbar. (You will be using it frequently)

Step 3: At the command prompt, CAREFULLY type ipconfig /all (NO typos) and press Enter key to view detailed information about your network connection settings.

Step 4: Take a screenshot of the entire Command Prompt window displaying your current network settings. (expand the window if necessary – make sure to get the entire output of the ipconfig /all command) SCORING: approx. 2 pts. per question, 4 points per screenshot. Partially correct questions/screenshots can earn half of the points.

- Screenshot of ipconfig /all command output in the Command Prompt window:

Step 5: Using the output generated from the ipconfig /all command in the Windows 10 VM (or your screenshot above), answer the following questions about your network connection settings below:

- What is your IPv4 address? 10.55.65.244 (answers vary)

- What is your subnet mask? 255.255.255.0

- What is your default gateway address? 10.55.65.254

- Look carefully at the various settings. Is your IPv4 address statically set or automatically configured? (Hint: look for configuration setting(s) that can be enabled/disabled) Automatically configured. Automatically assigned – DHCP and Autoconfiguration enabled.

- How long is your IPv4 address leased for (approx. number of hours)? 4.0 hours (answers vary)

- Look at the Description of your ethernet adapter (Network interface card/NIC). What is its make and model? Intel 82574L Gigabit Network Connection

- Pg. 342 of your textbook describes a MAC address, also called a “physical address”. What is your NIC’s MAC address? 00-50-56-9b-93-26 (answers vary)

- How many DNS server addresses are configured? two

Step 6: Review pg. 423, item #4 in your textbook to view the commands to release and renew your IP address. In your Windows 10 VM Command Prompt window, run the command to RELEASE your IP address ONLY (do NOT renew at this time).

- What command did you enter? ipconfig /release

Step 7: Type ipconfig /all into the Command Prompt window. Take a screenshot of the output generated by the command:

- Screenshot of ipconfig /all command output in the Command Prompt window:

Step 8: Answer the following questions about the results of releasing your IP address:

- What is your IPv4 address now? ? 169.254.176.94 (answers vary)

- View pg. 422 Figure 8-51 of your textbook. What is the significance of a 169.254.x.x address (what does it indicate)? Indicates an APIPA IP address; you were not able to lease an IP address, etc. answers vary.

- What is your subnet mask now? 255.255.0.0

- What is your default gateway address? Blank/doesn’t exist

- Has your physical (MAC) address changed? No

- Type ping 8.8.8.8 in the Command Prompt window to ping Google’s DNS server and test network connectivity. What error message is displayed? Ping: transmit failed. General Failure. x4

- Launch the Edge browser from the Windows 10 VM taskbar. Click and wait patiently ( it takes a minute or two) and click “No thanks” if you see a popup message. After the page loads, what error message is displayed? You’re not connected…let’s get you back online (or similar)

Step 9:

FIRST: Close the Edge browser window to return to the Command Prompt window. Warning: if the Edge Browser won’t close, click just click on the Command Prompt icon on your taskbar to relaunch its window. Don’t worry about the browser; it will eventually close.

SECOND: Review pg. 423, item #4 in your textbook to view the command to renew your IP address. Run the command to RENEW your IP address ONLY. WARNING: BE PATIENT - it takes a minute.

- What command did you enter? ipconfig /renew

Step 10: Type ipconfig /all into the Command Prompt window. Take a screenshot of the output generated by the command:

- Screenshot of ipconfig /all command output in the Command Prompt window:

Step 11: Answer the following questions about the results of renewing your IP address:

- What is your IPv4 address now? 10.55.65.244 (answers vary)

- Is it the same or different than your ORIGINAL IP address in Step 5? The same.

- Look at your screenshot in Step 4. Has your lease time changed (not the overall time but the actual time of day it was obtained and expires)? Yes

- Type ping 8.8.8.8 in the Command Prompt window to ping Google’s DNS server and test network connectivity. What connectivity message is displayed? Reply from 8.8.8.8: bytes=32 time=6ms TTL=52 x4 (or similar)

- Launch the Edge browser from the Windows 10 VM taskbar. Do you have network/Internet connectivity? yes

Step 12: Close the Edge Browser and Command Prompt window.

2. Custom Project 2: Changing Adapter Settings and Setting a Static IP address

Review pgs. 338-339 on how to configure your network adapter.

Step 1: Right-click the Windows 10 VM Start icon and click Network Connections. Click on Change adapter options.

Step 2: Right-click Ethernet0  click Properties from the drop-down menu.

Step 3: In the Ethernet0 Properties window, double-click Internet Protocol Version 4 (TCP/IPv4) to launch it.

- How are you currently set to receive your IP AND DNS server address? Obtaining both automatically

Step 4: Click Use the following IP address.  Choosing this option informs the network that you are going to statically (manually) assign the IP address for this client.

Step 5: For the IP address on your Ethernet Network Adapter (NIC), CAREFULLY enter 192.168.1.10

Step 6: Enter 255.255.255.0 for the subnet mask

Step 7: Leave “Default Gateway”, “Preferred DNS server”, and “Alternative DNS server” BLANK.

Step 8: Take a screenshot of the Internet Protocol Version 4 (TCP/IPv4) Properties window displaying your static IPv4 settings.

- Screenshot:

Step 9: Close the open dialog boxes by clicking OK TWICE (and Close if necessary) ONLY your Network Connections window should still be open.

WARNING  IF YOU DO NOT CLOSE ALL NETWORK CONFIGURATION WINDOWS BY CLICKING OK, YOUR IP ADDRESS WILL NOT BE CHANGED/SET TO THE NEW ADDRESS.

Note: Because we are in a managed VM environment that we do not control, our IP address has not actually changed to the address we just statically set. Therefore, we will NOT bother verifying it in the Command Prompt window. Normally you WOULD verify it by running another ipconfig /all.

Step 10: Return to AND repeat steps 1-3 of THIS project (above) to reset your Ethernet adapter back to Obtaining BOTH an IP and DNS Server addresses automatically. (change BOTH radio buttons back)

Step 11: Close the open dialog boxes by clicking OK TWICE (and Close if necessary) ONLY your Network Connections window should still be open.

Step 12: Return to the Windows 10 VM Command Prompt window (relaunch from the taskbar if necessary).

Step 13: Type ping google.com into the Command Prompt window to verify you can ping successfully, meaning you have connectivity.

  - If you CANNOT successfully ping Google, troubleshoot BEFORE proceeding.

  - FYI – you should receive 4 replies from an IP address belonging to Google. General Failure is NOT a successful ping.

- What IP address (belonging to Google) are you receiving replies from? 216.58.194.206 (answers vary)

Step 14: Close all Network Connections windows, and if necessary, browser windows. Just the Command Prompt window should be left open for the next project.

3. Custom Project 3: Pipe the output of troubleshooting commands to a text file

Review pg. 366 ipconfig /displaydns & ipconfig /flushdns commands; review pg. 369 NETSTAT command that includes the “>>” description. Warning: a single “>” carrot (right-facing arrow/greater-than sign) OVERWRITES the original text file each time its run, while a double “>>” carrot (right-facing arrow/greater-than sign) APPENDS the text file, adding the new entry to the end of the file.

Step 1: Inside the Windows 10 VM Command Prompt window (relaunch from the taskbar if necessary), type ipconfig /displaydns and press Enter to display all stored DNS entries. How many records are stored (scroll up and count each record name).

- Approximately how many records are listed? (count the approx. number of “record blocks” - NO need to be exact if you encounter a lot): 42 (Answers vary)

Step 2: CAREFULLY type cd Desktop at the command prompt and press Enter to change to change your current working directory to the Desktop. This step lets us save to the Desktop without having to specify the path to it in our command. (Notice how the prompt changed: C:\Users\Student\Desktop> is where we’re currently located in the file system while working in the command-line window)

Step 3: Type ipconfig /displaydns >> troubleshooting.txt which will automatically create, name, and save “troubleshooting.txt” text file to the Windows 10 VM Desktop that has the generated output of the ipconfig /displaydns command saved within.

Command clarification:

ipconfig[space]/displaydns[space]>>[space] troubleshooting.txt

Step 4: Minimize the Command Prompt window, and locate the new troubleshooting.txt file that just appeared on your Desktop. Open the file and quickly scroll through it. Then take a screenshot displaying its contents (not all of the entries, just make sure two or so records are showing).

- Screenshot of OPEN text file troubleshooting.txt:

Step 5: You MUST CLOSE the troubleshooting.txt text file BEFORE proceeding to step 6.

Step 6: Using ONE carrot (right-facing arrow/greater-than sign) instead of two this time, type ipconfig /flushdns > troubleshooting.txt to OVERWRITE the displaydns information written earlier to the “troubleshooting.txt” text file.

Command clarification:

ipconfig[space]/flushdns[space]>[space] troubleshooting.txt

Step 7: Locate the troubleshooting.txt file on the Desktop and reopen it. Take a screenshot displaying its modified contents.

- Screenshot of OPEN text file troubleshooting.txt:

Step 8: You MUST CLOSE the troubleshooting.txt text file BEFORE proceeding to step 9.

Step 9: Using TWO carrots (right-facing arrows/greater-than signs) instead of one this time, type netstat >> troubleshooting.txt to APPEND or ADD the Ethernet Network Adapter’s Active Connections information to the “troubleshooting.txt” file BENEATH the output of the ipconfig /flushdns command.

Command clarification:

netstat[space]>>[space] troubleshooting.txt

Step 10: Locate the troubleshooting.txt file on the Desktop and reopen it. Take a screenshot displaying its modified contents.

- Screenshot of OPEN text file troubleshooting.txt:

SUBMIT one word processed document to Canvas under WK4CH7-8 Lab, named yourfirst initial+lastname+WK#CH#Lab.docx

(Ex. amurphyWK4CH7-8Lab.docx)